23-NTU-CS-1010

Ali Faisal

BSCS-B-5

Question 1

1. What is the purpose of WebServer server(80); and what does port 80 represent?

Ans .  Webserver server(80) creates a web server object that listens for HTTP requests on port 80

webserver is a ESP32 library class that creates a HTTP server . Server is the name of server while port 80 is default HTTP port

Port 80 is a default port for HTTP communication so when you type ip address in browser itt opens in port 80

2. Explain the role of server.on("/", handleRoot); in this program

Ans . server.on("/", handleRoot); tells the ESP32 what action to take when a client opens the root webpage (/).
The / symbol represents the main page of the website, and handleRoot() is the function that is executed when this page is requested.
This function typically sends an HTML response to the browser.

3. Why is server.handleClient(); placed inside the loop() function? What will happen if it is removed?

server.handleClient(); checks for incoming client requests and processes them.

It must be inside loop() because:

- ESP32 runs continuously

- Web requests can arrive at any time

If server.handleClient(); is removed:

- The ESP32 will not respond to any browser requests

- The webpage will not load

- The server becomes inactive

### 4. In handleRoot(), explain the statement:

server.send(200, "text/html", html);

- This line sends the webpage to the browser.

- 200 → means the request is successful

- "text/html" → tells the browser it’s an HTML page

- html → the actual webpage content (temperature, humidity, etc.)

- One-line answer (exam friendly):
server.send(200, "text/html", html); sends the HTML page to the browser with a successful response.

### 5. What is the difference between displaying last measured sensor values and taking a fresh DHT reading inside handleRoot()?

| Last Measured Values | Fresh DHT Reading |
| --- | --- |
| Uses previously stored sensor data | Reads sensor every time page is loaded |
| Faster webpage loading | Slower response |
| No sensor delay | DHT sensor needs time to read |
| More stable web server | Can cause blocking / timeout |
| Recommended approach | Not recommended inside web handler |

LONG QUESTION

## ESP32 Webserver Temperature & Humidity Monitoring System – Working

### 1. ESP32 Wi-Fi Connection and IP Address Assignment

At startup, the ESP32 connects to Wi-Fi using WiFi.begin(ssid, password).

It continuously checks WiFi.status() until the connection is successful.

Once connected, the router assigns an IP address to the ESP32, accessible on the local network.

This IP address is printed to the Serial Monitor and displayed on the OLED.

Users can type this IP in a browser to access the web page.

### 2. Web Server Initialization and Request Handling

A web server object is created using:

WebServer server(80);

server.on("/", handleRoot); maps the root URL / to the handleRoot() function.

server.begin(); starts the web server.

In the loop(), server.handleClient(); continuously checks for incoming browser requests and responds with the appropriate webpage.

Without server.handleClient(), the server would not respond.

### 3. Button-Based Sensor Reading and OLED Update

A physical button is connected to a GPIO pin (e.g., pin 5) with INPUT_PULLUP.

The ESP32 monitors the button in loop().

When a falling edge is detected (button pressed), the ESP32:

Reads temperature and humidity from the DHT22 sensor (readDHTValues()).

Updates global variables lastTemp and lastHum.

Displays the readings on the OLED screen (showOnOLED()).

This avoids frequent slow DHT readings on every webpage request.

### 4. Dynamic HTML Webpage Generation

The handleRoot() function generates an HTML page as a string.

It dynamically includes:

Temperature (lastTemp)

Humidity (lastHum)

The webpage shows a message if no valid data is available yet.

The page instructs the user to press the physical button to update readings.

server.send(200, "text/html", html);

This sends the HTML page to the browser with HTTP status code 200.

### 5. Purpose of Meta Refresh in the Webpage

<meta http-equiv='refresh' content='5'>

This tells the browser to automatically refresh the page every 5 seconds.

Users see updated readings without manually reloading the page.

Works well with the last measured values, avoiding slow sensor reads on every refresh.

### 6. Common Issues and Solutions in ESP32 Webserver Projects

| Issue | Cause | Solution |
| --- | --- | --- |
| Webpage doesn’t load | server.handleClient() missing or Wi-Fi not connected | Ensure server.handleClient() is in loop, check Wi-Fi credentials |
| DHT sensor reads fail | Sensor slow or misconnected | Use pull-up resistor, read sensor periodically, not in handleRoot() |
| OLED shows nothing | Incorrect I2C address or wiring | Verify OLED wiring, use correct address 0x3C |
| Multiple page refreshes freeze server | Fresh sensor reads on each request | Store last values, refresh page with meta tag instead |
| Button debounce issues | Mechanical bouncing | Use small delay (~50 ms) after button press |

### 7. Complete Workflow

Boot: ESP32 starts, initializes OLED, connects to Wi-Fi.

IP Assigned: ESP32 IP shown on OLED and Serial Monitor.

Web Server: Initialized on port 80, root URL mapped to handleRoot().

User Request: Browser requests /, ESP32 sends HTML page with last measured values.

Button Press: User presses button → ESP32 reads DHT → updates OLED → global variables updated.

Auto Refresh: Webpage refreshes every 5 seconds to show latest readings.

### 1. What is the role of Blynk Template ID in an ESP32 IoT project? Why must it match the cloud template?

Blynk Template ID uniquely identifies the cloud template assigned to the device.

- It tells Blynk which widgets, virtual pins, and settings the device should use.

- If the Template ID does not match, the ESP32 cannot communicate properly with the cloud, and the dashboard/widgets will not work.

### 2. Differentiate between Blynk Template ID and Blynk Auth Token.

| Blynk Template ID | Blynk Auth Token |
| --- | --- |
| Identifies the cloud template for the device | Authenticates the specific device to the cloud |
| Same for all devices using that template | Unique for each device |
| Used for device setup and dashboard mapping | Used to send/receive data securely |

3. Why does using DHT22 code with a DHT11 sensor produce incorrect readings? Mention one key difference between the two sensors.

DHT22 and DHT11 have different data timing and ranges.

- DHT11: Temperature 0–50°C, Humidity 20–80%, slower response.

- DHT22: Temperature -40–80°C, Humidity 0–100%, higher precision.

- Using DHT22 code on a DHT11 may produce incorrect values or NaN readings.

4. What are Virtual Pins in Blynk? Why are they preferred over physical GPIO pins for cloud communication?

- Virtual Pins (V0, V1, …) are software-defined pins in the Blynk app.

- They allow sending/receiving data without using physical GPIO pins.

- Preferred because:

- Multiple widgets can use the same pin

- Reduces hardware limitations

- Safer for cloud communication

5. What is the purpose of using BlynkTimer instead of delay() in ESP32 IoT applications?

- delay() blocks code execution, stopping sensor reads, button detection, and Blynk communication.

- BlynkTimer allows non-blocking periodic tasks, like reading sensors or sending data, without freezing the ESP32.

Long Question

Explain the complete workflow of interfacing ESP32 with Blynk Cloud to display temperature and humidity values.

Your answer should include:

- Creation of Blynk Template and Datastreams

- Role of Template ID, Template Name, and Auth Token

- Sensor configuration issues (DHT11 vs DHT22)

- Sending data using Blynk.virtualWrite()

- Common problems faced during configuration and their solutions

## ESP32 + Blynk Cloud – Temperature & Humidity Workflow

### 1. Creation of Blynk Template and Datastreams

- In the Blynk cloud dashboard, create a new Template for the device.

- Add Datastreams (Virtual Pins) to the template, e.g.:

- V0 → Temperature

- V1 → Humidity

- Datastreams allow the ESP32 to send live sensor readings to the cloud and display them in widgets.

- Templates define widgets, pins, and device behavior.

### 2. Role of Template ID, Template Name, and Auth Token

- Template ID: Uniquely identifies the Blynk cloud template.

- The ESP32 must use the exact Template ID to map to the correct cloud dashboard.

- Template Name: Human-readable name of the device template (used for organization).

- Auth Token: Device-specific key used to authenticate the ESP32 with Blynk cloud.

- Without the correct Auth Token, data cannot be sent or received.

Summary: Template ID → template mapping, Auth Token → device authentication.

### 3. Sensor Configuration Issues (DHT11 vs DHT22)

- DHT22 has a wider range and higher precision than DHT11:

- DHT22: -40 to 80°C, 0–100% RH

- DHT11: 0 to 50°C, 20–80% RH

- Using DHT22 code with DHT11 causes incorrect readings because the timing and precision are different.

- Proper sensor selection ensures accurate cloud data.

### 4. Sending Data Using Blynk.virtualWrite()

- ESP32 reads sensor values from the DHT sensor.

- Values are sent to the cloud using:

- Blynk.virtualWrite(V0, temperature);

- Blynk.virtualWrite(V1, humidity);

- Virtual Pins correspond to datastreams in the cloud.

- Widgets in the Blynk app (e.g., gauges, value displays) automatically update with new readings.

- Optionally, a timer can be used to send data periodically without blocking code execution.

### 5. Common Problems and Solutions

| Problem | Cause | Solution |
| --- | --- | --- |
| ESP32 cannot connect to Blynk | Wrong Auth Token or Wi-Fi credentials | Verify credentials, copy correct token |
| Dashboard widgets not updating | Wrong Template ID or Virtual Pin mismatch | Check Template ID and Virtual Pin assignments |
| Sensor readings incorrect | DHT11 vs DHT22 mismatch | Use correct sensor type in code |
| Multiple readings freeze ESP32 | Using delay() for periodic updates | Use BlynkTimer for non-blocking tasks |
| No OLED display | Wrong I2C pins or address | Verify SDA/SCL and OLED address (0x3C) |

### 6. Overall Workflow

- ESP32 Setup: Initialize Wi-Fi, OLED, button, and DHT sensor.

- Blynk Initialization: Use Template ID and Auth Token to connect to cloud.

- Sensor Readings: Read DHT values manually (button) or periodically (BlynkTimer).

- Data Sending: Send temperature and humidity to Blynk virtual pins using Blynk.virtualWrite().

- Cloud Display: Widgets in Blynk app display live values.

- Error Handling: Check sensor errors, network issues, or template mismatches.